Software Design Document (SDD)

Homecare Management System (HMS)

Version 1.0

1. Introduction

1.1 Purpose

This Software Design Document provides a high-level architectural overview of the Homecare Management System. It describes the system components, their interactions, and the technology choices made to fulfill the requirements outlined in the SRS.

2. Architectural Overview

The HMS will adopt a hybrid architecture combining edge computing for low-latency safety responses and cloud computing for data aggregation, analytics, and accessibility .

- Edge Tier: Resides in the client's home. Responsible for real-time data collection from sensors and executing immediate alerts even without internet.

- Cloud Tier: Centralized server. Responsible for data storage, complex analytics (AI behavioral analysis), and serving the web dashboards to caregivers .

- Client Tier: End-user devices (Smartphones, Web Browsers).

High-Level Architecture Diagram (Conceptual):
[For your portfolio, you would draw a diagram here. The description is as follows:]
[IoT Sensors] <--> [Local Hub / Raspberry Pi] <--> (Encrypted Cloud API) <--> [Cloud Server (Database + DSS)] <--> [Caregiver Mobile/Web App]

3. Component Design

3.1 Sensor System (Hardware/Embedded)

- Components: ESP32 microcontrollers, BLE Beacons, Load Cell sensors (bed), Magnetic Door sensors, Wearable watch with accelerometer .

- Function: Collect raw data (movement, fall events, door open/close) and transmit it via Wi-Fi/MQTT to the Local Hub. Design focuses on unobtrusiveness and low power consumption .

3.2 Local Hub / Edge Processor (Raspberry Pi 4)

- Function:

  - Data Aggregation: Collects data from all local sensors.

  - Edge Processing: Runs local instances of the Decision Support System (DSS). For example, if a fall is detected and the internet is down, the hub triggers a local siren or a direct alert to a local base station .

  - Communication Broker: Manages MQTT/HTTP communication between devices and relays filtered data to the cloud.

3.3 Cloud Server & Database

- Application Server: Hosts the business logic (RESTful APIs) for user management, billing, and care plan scheduling.

- Database: Utilizes a hybrid approach. Time-series database (e.g., InfluxDB) for storing sensor data (temperature, heart rate trends) and Relational Database (e.g., PostgreSQL) for user profiles, billing, and care plans .

- Decision Support System (DSS): A software module that analyzes long-term data to identify trends (e.g., decline in mobility) and generates high-level behavioral insights for caregivers .

3.4 Client Applications

- Caregiver Web Dashboard:

  - Technology: React.js or similar framework.

  - Features: "Bird‘s eye view" of client status, visit scheduling, billing management, and document management .

- Caregiver Mobile App:

  - Technology: React Native (cross-platform).

  - Features: Receives push notifications for alerts, views real-time location, and checks medication adherence .

- Client Wearable Interface:

  - Technology: Proprietary firmware on smartwatch.

  - Features: One-touch alarm, vibration reminders for medications. The interface is kept minimal for ease of use .

4. Interface Design

- IoT Communication Protocol: MQTT (Message Queuing Telemetry Transport) will be used for lightweight communication between sensors and the local hub .

- API Protocol: RESTful HTTPS/JSON for communication between the Local Hub, Cloud Server, and Mobile Apps.

- Hardware API Integration: The system will include a compatibility layer to integrate with third-party APIs (e.g., eHealth records, Mindose dispensers) .

5. Security Design

- Data at Rest: All Personal Identifiable Information (PII) and health records in the cloud database will be encrypted using AES-256.

- Data in Transit: All external network traffic will use TLS 1.3.

- Authentication: OAuth 2.0 for web and mobile applications, potentially with multi-factor authentication for professional caregivers.